Quiz Practice

- Consider the periodic signal given in the figure below.

- Is this signal finite support or infinite support?

- Is this signal causal or not cauisal?

- Is this signal even, odd or neither?

- What is the period of the signal?

- Calculate the energy of the signal.

- Calculate the power of the signal.

- The trigonometric Fourier series is given by

xt= c0+2k=1∞[ckcoskΩ0t+dksinkΩ0t]

Calculate the DC term.

Write the integral expressions for ck and dk

Do not carry out the calculations for ck and dk

- Consider the suystem given by:

yt=Sxt=t+3cost-3x(t)

- Is the system linear?

- Is the system time-invariant?

- Is the system cauysal?

- Is the system BIBO stable?

- Consider the causal LTI system with impulse response h(t) and transfer function given by:

Hs=s+3s(s-1)

- What are the zeros?

- What are the poles?

- Is the system BIBO stable?

- If the input to the system is x(t) = cos(t) u(t) where u(t) is the unit-step, find the output y(t).